The KGN Pillinger External CAT 5 Washdown Booster Set is designed to prevent backflow contamination of the mains water supply, while maintaining flow and pressure. The unit is compact, lightweight, and housed in a GRP cabinet. Suitable for external use, the unit also has a built in heater for cold weather protection.

Advantages

• Automatic operation

• Simple to wall or floor mount and install

• Easy access and maintenance

• Suitable for external usage

Design

Single peripheral pump and water storage

tank housed in a GRP cabinet.

1 outlet connection only

5 metre hose limitation

Applications

• Bin store washdown

• Limited boosting applications

Features

• Stand stock item

• Duty: 0.25 l/s @ 2.2 bar (mid curve)

• Lightweight GRP Kiosk

• Break Tank: 18 litre (4 gallon)

• Cabinet fitted with a thermostat controlled heater

• CAT5 AB Air gap to prevent backflow contamination

Dimensions (approx.) Electrical Approx Weight

Width: 610 mm Watts: 0.37 kW 30 kg

Depth: 390 mm Volts: 240 V

Height: 1070 mm Amps: 2.3 A