The KGN Pillinger “E” Electronic range of Pressurisation Units are self-contained, fully automatic, PLC controlled and designed to maintain the required systems pressure in both heating and chilled water systems.

Advantages of a sealed system

- Elimination of corrosion due to the lack of contact between air and water within the system

- Automatically pumps water into the system(s) to maintain the correct set pressure

- Eliminates the need for a large tank within the roof space of a building

Design

All incorporate bronze pumps which are housed in a brushed 304 stainless steel
cabinet suitable for floor standing or wall mounting.

SE300E:	Twin pump single system

Features

- PLC controller

- Heavy duty fill pumps

- Pipework: Nylon push fit

- Pump protection: MCB(s)

- Pressure regulating valve

- Tank low level cut off switch

- High and low pressure output

- Break tank: One 18 litres (4 gallons)

- Designed for system pressures up to 6 bar

- Automatic pump duty changeover per operation

- Additional expansion vessel(s) can be calculated and supplied upon request

- Pre-assembled with high and low pressure cut out VFC terminals for boiler and chiller interface

- Common VFC for unit fault

- Visual Indications: Illuminated mains on switch and visual readout on PLC display.

- Polypropylene tank incorporating air gap,                ball/float valve (DN15 BS21) and overflow (DN22 BS21)

Dimensions                           Electrical                                            Weight

Width:  420 mm                      Watts:   0.75 kW                                38.5 kg                                                                                                                                                                            Depth:  370 mm                      Volts:    240 V      or     400 V                                                                                                                                                                                            Height: 750 mm                      Amps:   3.41 A     or    1.59 A